A

DISSERTATION REPORT

ON

Hybrid IoT and Computer Vision Framework for Real-Time Industrial Hazard Detection and Worker Safety Compliance

Under the supervision of

Supervisor

Dr. Umesh Dutta

CEO of MRIIF

Co-Supervisor

Dr. Abhiruchi Passi

HOD, Department of ECE

Submitted by:

Sushant Tomar

1/24/SET/MVLSI/001

Submitted to:

Department of ECE

Department of Electronics and Communication Engineering

School of Engineering and Technology

Manav Rachna International Institute of Research & Studies

Sector-43, Suraj Kund–Delhi Road, Faridabad– 121004

CERTIFICATE

This is to certify that the project proposal titled “Hybrid IoT–Computer Vision Framework: An AI-Driven Approach for Real-Time Industrial Hazard Detection and Worker Safety Compliance” submitted by Sushant Tomar, Roll No: 1/24/SET/MVLSI/001, is a record of bona fide work carried out under my supervision.

Supervisor:
Prof. (Dr.) Umesh Dutta, CEO of MRIIF

Co-Supervisor:
Prof. (Dr.) Abhiruchi Passi, Department of ECE

DECLARATION

I, “Sushant Tomar” student of III sem, M. Tech ECE, studying at Manav Rachna International Institute of Research & Studies, Faridabad, hereby declare that the project report on “Hybrid IoT and Computer Vision Framework for Real-Time Industrial Hazard Detection and Worker Safety Compliance” submitted to Department of ECE, MRIIRS, Faridabad is the original work conducted by me. The information and data given in the report is authentic to the best of our knowledge. This project report is not being submitted to any other University for award of any other Degree.

(Sushant Tomar & RN: 1/24/SET/MVLSI/001)

This is to certify that the above statement made by the candidate is correct to the best of my knowledge. This report has been reviewed by me and submitted with my approval as a supervisor assigned to the student.

Name of the supervisor                            Signature                                            Date

Dr. Umesh Dutta                              _____________                               29-Nov-2025

Name of the Co- supervisor                            Signature                                      Date

Dr. Abhiruchi Passi                            _____________                             29-Nov-2025

ACKNOWLEDGEMENT

It is my pleasure to be indebted to various people, who directly or indirectly contributed in the development of this work and who influenced my thinking, behavior, and acts during the course of study. I express my sincere gratitude to Dr. H. C. Rai, worthy Dean, MRIIRS for providing us an opportunity to undergo this project work “Hybrid IoT and Computer Vision Framework for Real-Time Industrial Hazard Detection and Worker Safety Compliance”

I am thankful to “Dr. Umesh Dutta, CEO of MRIIF” his support, cooperation, and motivation provided to me during the work, and for constant inspiration, presence and blessings. I also extend my sincere appreciation to “Dr. Abhiruchi Passi, HOD, Dept of ECE”. who provided her valuable suggestions and precious time in accomplishing this project report.

Lastly, I would like to thank the almighty and our parents for their moral support.

(Sushant Tomar & RN: 1/24/SET/MVLSI/001)

Abstract

This dissertation proposes a hybrid framework that integrates Internet of Things (IoT) sensing and Computer Vision (CV) to enable real-time detection of industrial hazards and ensure worker safety compliance. The system is designed to monitor both environmental and behavioral risk factors by combining data from distributed IoT sensors (e.g., gas, temperature, vibration) with visual analytics for detecting personal protective equipment (PPE) violations, unsafe proximity to machinery, and fall incidents. Unlike traditional siloed systems, the proposed architecture emphasizes sensor–vision fusion at the edge to reduce latency, enhance detection accuracy, and minimize false alarms. The framework will be deployed on edge computing platforms to ensure privacy-preserving, low-latency inference, and will utilize lightweight deep learning models optimized for constrained hardware. A modular dashboard and MQTT-based alerting mechanism will provide real-time feedback to safety personnel. The research will follow a phased methodology encompassing system design, dataset collection, model training, fusion logic development, and performance evaluation. The anticipated outcome is a validated prototype capable of operating in semi-operational industrial environments, offering a scalable and intelligent solution for Industry 4.0 safety automation.

Keywords: Industrial Safety, IoT, Computer Vision, Edge Computing, PPE Detection, Sensor Fusion, Real-Time Monitoring, Industry 4.0

| Table of Contents S. No.; Topic; Page No.. 1; Introduction; 7. 2; Literature Survey; 12. 3; Research Methodology; 16. 4; System Implementation and Development; 24. 5; Results and Discussions; 30. 6; Conclusions; 31. 7; Limitations; 33. 8; Suggestions and Future Scope; 34. 9; References/Bibliography; 35. |  |  |
| --- | --- | --- |

CHAPTER 1: INTRODUCTION

  - Background to Study

Industrial environments such as manufacturing plants, construction sites, logistics facilities, and mining operations continue to remain inherently hazardous despite continuous technological advancements. Workers are commonly exposed to multiple risk factors including toxic gases, high temperatures, heavy machinery, fast-moving equipment, slippery surfaces, and unstable working conditions. These hazards contribute to accidents, injuries, and fatalities across industries. According to global safety reports, industrial incidents remain one of the leading causes of workplace-related harm, highlighting significant gaps in traditional monitoring systems [1][3][8].

Historically, worker safety monitoring has relied on manual supervision and passive tools such as CCTV cameras. Both methods suffer from major shortcomings. Manual supervision is limited by human fatigue, narrow field of view, delayed reaction time, and susceptibility to oversight. CCTV systems, while widely used, only provide passive surveillance and require continuous human attention. They cannot automatically identify hazardous conditions, detect PPE non-compliance, or spot unsafe behavior in real time.

The emergence of Industry 4.0 has introduced new opportunities to enhance industrial safety through the integration of technologies such as the Internet of Things (IoT), Artificial Intelligence (AI), Machine Learning (ML), Computer Vision (CV), and Edge Computing. IoT sensors allow real-time monitoring of environmental conditions such as gas concentration, heat levels, humidity, and equipment vibration. Computer Vision, powered by deep learning, enables automated detection of PPE compliance, unsafe worker behavior, falls, slips, and restricted zone violations.

However, existing systems typically operate as isolated modules: IoT-only systems detect environmental anomalies but cannot recognize visual cues, whereas CV-only systems detect visual hazards but cannot sense invisible environmental threats. These fragmented systems create blind spots, increase the probability of false positives or missed detections, and fail under challenging industrial conditions such as poor lighting, occlusion, dust, and sensor drift.

To address these gaps, this research proposes a Hybrid IoT–Computer Vision Framework that fuses environmental sensor data with visual analytics using edge computing. This integrated approach improves real-time hazard detection, reduces latency, minimizes false alarms, and ensures privacy-compliant processing while supporting predictive safety management aligned with Industry 4.0 objectives.

1.2 Problem Statement

Violations or unsafe worker behavior. Computer Vision systems, although capable of analyzing visual indicators, cannot detect environmental hazards such as gas leakage or humidity spikes. These single-modality systems operate independently and fail to provide a complete situational understanding.

Additional challenges arise when cloud-based processing is used. Cloud systems introduce latency, which can be dangerous during time-sensitive hazard scenarios. Furthermore, transmitting raw video frames to cloud servers raises privacy concerns and increases network dependency.

There is currently no unified solution capable of integrating IoT and computer vision at the edge for comprehensive, low-latency, and privacy-preserving real-time safety monitoring. The core problem addressed in this study is the absence of a hybrid, intelligeDespite major improvements in industrial process automation and safety standards, workplace accidents remain a persistent issue due to limitations in current monitoring mechanisms. Industrial incidents related to equipment malfunction, toxic gas leakage, thermal hazards, and unsafe human behavior continue to lead to injuries, fatalities, and significant operational disruptions. The shortcomings of existing monitoring systems include fragmented operation, lack of intelligent decision-making, high dependency on human supervision, and slow response times [1][7][12].

Traditional CCTV-based systems provide video surveillance but lack automated real-time analytics. IoT-only systems can detect invisible hazards such as gas concentration changes or thermal anomalies but cannot detect PPE nt, and efficient system capable of fusing IoT sensor data with computer vision detections to provide reliable industrial hazard identification and worker safety compliance.

3.3 Aim / Purpose of the Study

The aim of this study is to design, develop, and evaluate a Hybrid IoT–Computer Vision Framework for real-time industrial hazard detection and worker safety compliance. The system integrates IoT environmental sensing with deep learning–based computer vision analytics to overcome the limitations of standalone monitoring systems. The framework emphasizes low-latency edge processing, multi-modal safety detection, and reliable decision fusion to enhance safety outcomes in industrial environments [3][4][11].

## 1.3.1 Proposed Specific Objectives

- To design a modular hybrid architecture combining IoT sensors, computer vision modules, edge processing units, and real-time alerting mechanisms to support deployment across various industrial environments [3][11][19].

- To develop lightweight and edge-optimized deep learning models capable of detecting PPE compliance, unsafe worker behavior, proximity to machinery, slips, falls, and restricted area violations.

- To implement a sensor–vision fusion engine that correlates IoT anomalies with visual detections using confidence scoring, triggered analysis, and late-decision fusion strategies to reduce false positives and improve reliability.

- To deploy a functional prototype system using edge devices (Jetson Nano, Google Coral TPU, ESP32), IoT sensors (gas, temperature, vibration), camera modules, MQTT communication, and a real-time monitoring dashboard.

- To evaluate system performance using metrics such as precision, recall, F1-score, end-to-end latency, false alarm rate, and operational robustness, and to compare hybrid performance with IoT-only and CV-only systems.

1.4 Research Questions

1. How can IoT sensors be used to monitor real-time environmental and equipment-related hazards in industrial settings? [3][6][12]

2. What level of accuracy can deep-learning-based computer vision models achieve in detecting PPE compliance and unsafe worker behavior?

3. How can sensor–vision fusion improves hazard detection reliability compared to standalone IoT or CV systems?

4. How does edge computing contribute to reducing latency and improving system responsiveness in real-time safety monitoring?

5. What performance improvements does the proposed hybrid system offer over traditional safety monitoring mechanisms?

1.5 Significance of the Study

The significance of this study lies in its ability to combine IoT sensing and computer vision into a unified safety monitoring system that addresses the limitations of existing solutions. Industrial significance includes real-time detection of gas leaks, temperature anomalies, PPE violations, and unsafe worker behavior. The system enhances compliance with established safety standards including OSHA and ISO 45001 while minimizing downtime and improving workplace productivity [5][8][10].

From an academic perspective, this research contributes a reproducible hybrid architecture, a sensor–vision fusion methodology, edge optimization strategies, and structured evaluation metrics that add to ongoing research in industrial automation and AI-enabled safety systems. At the policy level, the study supports the development of AI-driven occupational safety frameworks and provides evidence-based insights for future regulatory advancements.

## 1.6 Scope of the Study

The scope of this research includes industrial workplaces such as manufacturing plants, warehouses, and construction sites where risk factors are prevalent. The study focuses on IoT sensors such as gas, temperature, and vibration monitoring devices, and computer vision models for PPE detection, unsafe behavior detection, and fall/slip identification. The system uses edge computing hardware such as Jetson Nano, Coral TPU, and ESP32 for local inference. Real-time alerts are transmitted through MQTT, and system performance is evaluated in controlled and semi-operational conditions [12][17].

## 1.7 Limitations of the Study

This study acknowledges certain limitations. Computer vision performance may degrade under low lighting, excessive dust, occlusion, or poor camera placement. IoT sensors may suffer from sensor drift, requiring periodic calibration to maintain accuracy. Edge devices have computational and thermal constraints that may limit model complexity. Network connectivity issues can delay MQTT-based alerts. Camera-based monitoring raises privacy concerns and requires appropriate anonymization measures and ethical data handling policies [17][18].

## 1.8 Organization of the Study

This dissertation is organized into five chapters.

Chapter 1 presents the introduction, background, problem statement, aim, objectives, significance, scope, and limitations of the study.

Chapter 2 provides a detailed literature review covering IoT-based safety systems, computer vision techniques, fusion models, and edge computing approaches.

Chapter 3 outlines the research methodology, including system architecture, design components, implementation details, and evaluation metrics.

Chapter 4 presents the results and discusses system performance, reliability, and comparative analysis.

Chapter 5 concludes the study and provides recommendations and future research directions.

# CHAPTER 2: LITERATURE SURVEY

## 2.1 Introduction

This chapter presents an in-depth review of existing literature related to industrial safety monitoring systems, IoT-based environmental sensing, computer vision for PPE and behavior detection, sensor–vision data fusion, and edge computing for real-time AI deployment. It highlights the historical development, foundational theories, contemporary approaches, and limitations in current technologies. The purpose of this review is to establish the research gap that motivates the development of a Hybrid IoT–Computer Vision Framework for real-time industrial hazard detection. The chapter concludes by identifying challenges and gaps that the proposed system aims to address [1][2][3].

## 2.2 Conceptual Review

### 2.2.1 IoT-Based Industrial Safety Monitoring

The Internet of Things (IoT) has become a central technology in Industry 4.0, enabling continuous monitoring of environmental and operational parameters. IoT sensors such as MQ gas sensors, DHT temperature and humidity sensors, vibration sensors, ultrasonic proximity sensors, and smoke detectors are widely used for safety-enhancing applications [4][5].

Studies have demonstrated the effectiveness of IoT in detecting gas leaks, temperature anomalies, equipment overheating, and machine vibration patterns that precede mechanical failures. IoT-enabled industrial monitoring systems benefit from:

- Low-power sensing networks

- Real-time telemetry

- Distributed data collection

- Threshold-based alerting mechanisms

However, IoT-only systems lack contextual awareness. They cannot determine whether workers are wearing PPE, following safety protocols, or entering restricted zones. Additionally, IoT sensors exhibit issues such as drift, calibration requirements, and susceptibility to environmental noise, limiting their standalone reliability [6][7].

### 2.2.2 Computer Vision for PPE and Unsafe Behavior Detection

Computer Vision (CV) has become a dominant technique in industrial safety automation due to advances in deep learning models such as YOLO, SSD, Faster R-CNN, and EfficientDet. These models can identify objects, segment regions, and detect human posture and actions in real time [8][9].

CV-based safety solutions include:

- Helmet, vest, glove, and boots detection

- Fall/slip detection using pose estimation

- Zone intrusion detection

- Unsafe proximity to moving machinery

- Motion and crowd analysis

Recent studies have shown that deep learning–based PPE detection achieves high accuracy under ideal conditions. However, its reliability decreases below:

- Low lighting

- Fast motion

- Occlusion

- Dust and industrial fog

- Poor camera placement

Moreover, vision-only systems cannot detect invisible hazards like gas, radiation, or oxygen depletion, making them insufficient as standalone safety systems [10][11].

### 2.2.3 Sensor–Vision Fusion Approaches

Hybrid multi-modal safety systems combine IoT data with visual analytics to create a robust, context-aware hazard detection environment. Sensor–vision fusion can occur at different levels:

- Early fusion: Combining raw data before feature extraction

- Mid-level fusion: Combining learned features

- Late fusion: Combining final detection outputs

Recent research demonstrates that fusion improves detection confidence and reduces false positives by correlating environmental anomalies with visual evidence. For example, a temperature spike detected by IoT sensors can be confirmed by visual smoke detection, increasing reliability [12][13].

Despite this, existing fusion systems often rely on cloud processing, resulting in latency, privacy risks, and high bandwidth usage.

### 2.2.4 Edge Computing for Real-Time Safety Monitoring

Edge computing plays a vital role in deploying AI for industrial environments. Devices like the Nvidia Jetson series, Google Coral TPU, and high-performance microcontrollers enable on-device inference without relying on cloud servers [14][15].

Advantages of edge-based safety systems:

- Low latency

- Privacy preservation

- Reduced bandwidth requirements

- Higher reliability in offline scenarios

Studies confirm that edge-deployed AI models, when compressed using quantization or pruning, can achieve near-cloud performance while maintaining real-time responsiveness. However, challenges such as thermal throttling, memory constraints, and power limitations are frequently reported.

### 2.2.5 Ethical, Security, and Privacy Considerations

The deployment of AI and IoT in workplaces introduces privacy concerns, especially involving worker surveillance. Literature emphasizes the need for:

- On-device anonymization

- Minimization of data storage

- Use of bounding boxes instead of raw video frames

- Ethical compliance with privacy regulations

Security-wise, IoT systems are vulnerable to spoofing, tampering, and network intrusion. Encrypted communication protocols, secure firmware updates, and intrusion detection mechanisms are recommended in prior studies [16][17].

## 2.3 Empirical Review

Empirical studies across multiple sectors demonstrate the growing importance of hybrid multimodal monitoring systems.

- IoT Gas and Temperature Monitoring Studies:
Multiple studies confirm the reliability of gas sensors for early detection of industrial hazards such as methane, CO, and volatile organic compounds. However, false alarms due to humidity or dust remain a concern [18][19].

- PPE Detection Model Studies:
Researchers have evaluated YOLOv5, YOLOv7, MobileNet-SSD, and EfficientDet for PPE compliance monitoring. While high accuracy is achieved in controlled lighting, performance drops significantly in real industrial conditions [20][21].

- Fall Detection and Unsafe Posture Analysis:
Pose-estimation models such as OpenPose and MediaPipe have been tested for identifying slips, falls, or ergonomic risks. These models achieve strong results but require substantial computational resources [22][23].

- Hybrid Systems:
Recent empirical studies propose IoT + CV fusion–based frameworks that outperform standalone systems, demonstrating better precision, recall, and reduced false positives. However, these systems mostly rely on cloud servers, leading to increased latency [24][25].

Overall, empirical literature supports hybrid multi-modal monitoring but highlights the need for edge-based, real-time, privacy-preserving solutions.

## 2.4 Identified Research Gaps

From the above literature, several gaps become clear:

- Lack of a unified hybrid framework:
Most solutions focus either on IoT-only or CV-only systems, not a fused architecture.

- High latency in cloud-based systems:
Real-time industrial safety requires edge computing, which is not widely implemented.

- Limited multimodal fusion algorithms:
Existing fusion models are simplistic and do not use advanced decision-level integration.

- Insufficient real-world validation:
Many studies rely on controlled datasets rather than real industrial conditions.

- Privacy issues remain unresolved:
Few studies integrate anonymization directly on edge devices.

- Dataset limitations:
Public datasets for industrial safety (PPE, falls, machinery proximity) are limited, leading to poor generalization.

These gaps justify the need for a robust, edge-deployed Hybrid IoT-Computer Vision Framework like the one proposed in this research.

## 2.5 Summary of the Chapter

This chapter reviewed literature related to IoT-based environmental monitoring, deep learning–based computer vision for PPE and behavior detection, sensor–vision fusion strategies, and edge computing for real-time processing. The review highlighted limitations in existing single-modality systems and the need for a unified, intelligent, low-latency hybrid approach for industrial safety monitoring. These observed limitations and gaps form the foundation of the research presented in subsequent chapters.

# CHAPTER 3: RESEARCH METHODOLOGY

## 3.1 Introduction

This chapter explains the overall research methodology adopted for developing the Hybrid IoT–Computer Vision Framework for real-time industrial hazard detection and worker safety compliance. It presents research design, system architecture, data acquisition strategies, model development pipeline, fusion logic, tools and technologies used, verification methods, and data analysis approach. The methodology is based on an engineering research model involving iterative development, implementation, testing, and evaluation in controlled industrial-like conditions [1][3][7].

## 3.2 Research Design

The research follows a design–develop–deploy–evaluate methodology. This approach is suitable for engineering-based projects where both hardware and software components must be integrated, optimized, and tested in real-world environments.

The research design includes:

- Problem Analysis:
Identifying limitations in IoT-only and CV-only safety systems, reviewing literature, and establishing the need for a hybrid approach.

- System Design:
Designing a modular architecture combining IoT sensors, computer vision models, and an edge computing node.

- Prototype Development:
Implementing IoT sensing units, training computer vision models, programming microcontrollers, and integrating MQTT-based communication.

- Fusion Engine Implementation:
Combining IoT and CV results through decision-level fusion algorithms.

- Deployment and Testing:
Running the system under controlled industrial-like conditions to evaluate performance.

- Performance Evaluation:
Assessing accuracy, latency, false alarm rate, and system robustness.

This methodology ensures systematic development and objective evaluation of the proposed hybrid safety framework.

## 3.3 System Architecture

The proposed architecture consists of four core layers:

### 3.3.1 IoT Sensing Layer

This layer includes environmental sensors such as:

- Gas Sensors (MQ-series)

- Temperature & Humidity Sensors (DHT-series)

- Vibration Sensors

- Smoke Detectors

These sensors collect real-time industrial data, detect anomalies, and transmit signals to the Edge Layer. Sensor calibration routines are implemented to improve data accuracy and minimize drift and false triggering [4][6].

### 3.3.2 Computer Vision Layer

The vision module uses deep learning models optimized for edge devices. Models detect:

- Helmets, vests, gloves (PPE detection)

- Unsafe posture

- Entry into restricted zones

- Slip and fall conditions

YOLO-based object detection models and pose estimation techniques form the backbone of the CV layer. The models are optimized using quantization and pruning to meet edge constraints.

### 3.3.3 Edge Processing Layer

This layer runs on devices such as:

- NVIDIA Jetson Nano

- Google Coral TPU

- Raspberry Pi 4 with NPU accelerators

Functions include:

- Real-time inference (IoT + CV)

- Fusion engine execution

- Local anonymization

- Triggering alerts

- Offline operation in case of network failure [12][15]

### 3.3.4 Communication & Dashboard Layer

MQTT is used for lightweight, low-latency message transmission. A dashboard (Node-RED / web dashboard) displays real-time:

- Sensor readings

- Vision detections

- Alerts and warnings

- System logs

## 3.4 Block Diagrams and Design Components

### 3.4.1 System Block Diagram

(Src: https://www.ijsat.org/papers/2025/4/9165.pdf)

The system block diagram consists of:

- IoT sensors connected to a microcontroller (ESP32)

- Camera module feeding images to the edge device

- Edge device performing inference and fusion

- MQTT broker relaying alerts

- Dashboard showing real-time data

- Storage module for logging events

### 3.4.2 Flowchart of System Workflow

- Sensor data collection

- Vision module inference

- Preprocessing and filtering

- Fusion logic decision

- Real-time alert generation

- Dashboard visualization

- Event logging and feedback loop

Textual Architecture Diagram

Computer Vision Unit (Camera + AI Model)

│

Edge Processing Device (ESP32 / Jetson)

┌──────────────┬───────────────┐

│                                                                                         │

IoT Sensors                                                               MQTT / Cloud

│                                                                                         │

└──────────────┬───────────────┘

│

Dashboard & Alerts

### 3.4.3 Tools and Technologies Used

Hardware:

- Jetson Nano, Coral TPU, ESP32

- Gas, temperature, vibration sensors

- Camera module (USB or CSI)

Software:

- Python, TensorFlow Lite, PyTorch

- OpenCV, MQTT, Node-RED

- Arduino IDE for microcontroller programming

- Edge TPU compiler for model optimization

## 3.5 Data Collection Sources

### 3.5.1 IoT Sensor Data

Sensor readings were collected from:

- Simulated industrial environments

- Controlled gas release tests

- Thermal load experiments

- Machine vibration patterns

These datasets helped define operational thresholds and anomaly conditions [12][17].

### 3.5.2 Computer Vision Data

CV data sources include:

- Public PPE datasets

- Custom images captured during testing

- Annotated real-world footage from industrial-like conditions

- Synthetic data augmentation

The dataset includes images representing various lighting conditions, PPE usage patterns, occlusions, and behavioral scenarios.

### 3.5.3 Ground Truth Labeling

Annotation tools such as LabelImg and CVAT were used for bounding boxes and key point labels. Multiple annotators validated labels to increase reliability [19].

## 3.6 Fusion Engine Implementation

The fusion engine is a key component of the hybrid approach.

### 3.6.1 Trigger-Based Fusion

If IoT anomaly > threshold, the system immediately triggers a higher-priority CV inference.
Example: High temperature triggers “check for smoke or flame.”

### 3.6.2 Confidence-Weighted Fusion

Final decision confidence (C_final):

C_final = (W_s * C_s) + (W_v * C_v)

Where:

- C_s = IoT sensor confidence

- C_v = Vision detection confidence

- W_s, W_v = modality weights

### 3.6.3 Rule-Based Fusion

Rules such as:

- If (gas leak detected AND no PPE mask) → High Risk

- If (temperature spike detected AND worker detected) → Equipment Overheat Warning

These rules combine multi-modal signals for safety-critical decision-making.

## 3.7 Verification and Quality Control

Multiple validation strategies ensure system reliability:

### 3.7.1 Cross-Validation for CV Models

- K-fold testing

- Precision, recall, and F1-score evaluation

- Testing across environmental variations (low light, glare, dust)

### 3.7.2 Sensor Calibration Tests

- Drift correction

- Environmental compensation

- Error margin analysis

### 3.7.3 Edge Performance Testing

- FPS (frames per second)

- On-device latency

- Thermal performance

- Uptime during continuous operation

### 3.7.4 End-to-End System Testing

Scenarios included:

- Gas leak simulation

- PPE violation cases

- Unauthorized zone entry

- Slip/fall events

- High-temperature zones

Performance was logged and analyzed.

## 3.8 Method of Data Analysis

The following metrics were used for performance evaluation:

- Accuracy: Correct detections / total detections

- Precision: True positives / (true positives + false positives)

- Recall: True positives / (true positives + false negatives)

- F1-score: Harmonic mean of precision and recall

- System Latency: Time from sensor/vision input to alert

- False Alarm Rate: Incorrect hazard alerts / total alerts

- Uptime: Operational stability during long-duration tests

Data analysis was done using Python libraries such as Pandas, NumPy, Matplotlib, and Scikit-learn [20][21].

## 3.9 Ethical Considerations

Ethical guidelines followed include:

- Data anonymization by masking faces

- No storage of raw video unless necessary

- Secure communication protocols

- Worker privacy compliance

- Ethical approvals for testing scenarios

- Ensuring no harm or real danger during experiments

## 3.10 Summary of the Chapter

This chapter presented the detailed methodology used for developing the Hybrid IoT–Computer Vision Safety Framework. It included research design, system architecture, components, tools, data collection strategy, fusion engine logic, verification processes, and analysis techniques. This methodology establishes the foundation for system performance evaluation, which is presented in Chapter 4.

# CHAPTER 4: SYSTEM IMPLEMENTATION AND DEVELOPMENT

## 4.1 Introduction

The implementation phase translates the conceptual framework of the hybrid IoT–Computer Vision system into a fully functional prototype capable of real-time industrial hazard monitoring. Unlike theoretical models, this phase requires careful integration of hardware, software, and communication systems under practical constraints such as limited computational resources, sensor inaccuracies, and real-time processing requirements. The primary objective of the implementation was to develop a robust system that can simultaneously process multimodal data streams, execute intelligent decision-making, and generate actionable alerts with minimal latency.

The system was designed using a modular approach to ensure scalability and maintainability. Each module—sensor acquisition, vision processing, fusion logic, and communication—was implemented independently and later integrated into a cohesive pipeline. This modular architecture allows future enhancements, such as replacing models or adding sensors, without affecting the overall system structure.

## 4.2 Hardware Implementation

The hardware architecture was carefully selected to balance performance, cost, and scalability. The ESP32 microcontroller serves as the primary data acquisition unit due to its integrated Wi-Fi capabilities and efficient handling of sensor inputs. Multiple sensors were interfaced with the ESP32, including gas sensors (MQ-series), temperature sensors (DHT11), and vibration sensors. These sensors were chosen to represent common industrial hazards and to provide diverse environmental data streams.

The edge computing device, such as NVIDIA Jetson Nano, acts as the processing hub. It receives sensor data from the ESP32 and video input from the camera module. The choice of Jetson Nano was driven by its GPU acceleration capabilities, which enable efficient execution of deep learning models on edge devices.

A critical aspect of hardware implementation involved ensuring reliable communication between components. Serial communication and Wi-Fi-based transmission were used to ensure continuous data flow. Sensor calibration was performed using baseline measurements to minimize drift and improve accuracy. Power management was also considered, particularly for long-term deployment scenarios.

## 4.3 Software Architecture

The software system was designed as a multi-threaded pipeline to handle parallel processing of sensor and vision data. The architecture consists of independent modules running concurrently to ensure real-time performance.

The system includes the following major software components:

- Sensor Data Acquisition Module

- Computer Vision Processing Module

- Fusion Engine

- Alert and Communication Module

Each module communicates through shared memory or message queues, ensuring synchronization without blocking execution.

## 4.4 Computer Vision Module

The computer vision module was implemented using a deep learning-based object detection pipeline. YOLOv5 was used due to its balance between speed and accuracy. The model was trained on PPE datasets and optimized for edge deployment.

Below is a more realistic and structured implementation:

This implementation demonstrates:

- Object-oriented design

- Real-time frame processing

- Structured detection output

4.5 IoT Sensor Module

The sensor module reads and preprocesses real-time data from multiple sensors. Noise filtering and thresholding are applied before forwarding data to the fusion engine.

import serial

import json

class SensorModule:

def __init__(self, port):

self.serial_port = serial.Serial(port, 115200)

self.data = {}

def read_sensor(self):

line = self.serial_port.readline().decode().strip()

try:

self.data = json.loads(line)

except:

pass

return self.data

def analyze(self):

alerts = []

if self.data.get("gas", 0) > 400:

alerts.append("GAS_LEAK")

if self.data.get("temp", 0) > 50:

alerts.append("HIGH_TEMP")

return alerts

4.6 Fusion Engine

The fusion engine integrates sensor and vision outputs using weighted decision-making and rule-based logic.

class FusionEngine:

def __init__(self):

self.weights = {"sensor": 0.6, "vision": 0.4}

def compute_confidence(self, sensor_score, vision_score):

return (self.weights["sensor"] * sensor_score +

self.weights["vision"] * vision_score)

def decide(self, sensor_alerts, vision_detections):

risk_level = "LOW"

if "GAS_LEAK" in sensor_alerts:

for det in vision_detections:

if det["class"] == "NO_MASK":

return "CRITICAL"

if any(det["class"] == "FALL" for det in vision_detections):

return "EMERGENCY"

return risk_level

4.7 System Integration

The final system integrates all modules into a real-time pipeline using multi-threading.

vision = VisionSystem("model.pt")

sensor = SensorModule("/dev/ttyUSB0")

fusion = FusionEngine()

while True:

sensor_data = sensor.read_sensor()

sensor_alerts = sensor.analyze()

vision_detections = vision.current_detections

decision = fusion.decide(sensor_alerts, vision_detections)

print("System Status:", decision)

## 4.8 Analysis of PPE Detection Results and Observed Limitations

During the experimental evaluation of the proposed hybrid system, it was observed that the Personal Protective Equipment (PPE) detection module occasionally produced limited or null outputs, particularly in the form of empty detection results or absence of PPE violation indicators. This behavior is primarily attributed to the type of deep learning model used during testing and the availability of domain-specific training data.

In the current implementation, the system is designed to utilize a custom-trained YOLO-based model capable of detecting PPE-related classes such as helmets, safety vests, and protective gear. However, in scenarios where the custom model file (ppe_model.pt) is not available, the system automatically falls back to a default YOLO model pretrained on the COCO dataset. While this fallback model is effective for detecting general objects such as persons, vehicles, and everyday items, it does not include specialized industrial safety classes such as “helmet,” “no helmet,” or “safety vest.”

As a result, the vision module may successfully detect the presence of a worker (classified as “person”) but fails to identify PPE compliance or violations. Consequently, the fusion engine does not receive sufficient semantic information to classify PPE-related risks, leading to outputs that may appear as zero or inactive for PPE conditions.

Another contributing factor is the reliance on label-specific logic within the fusion engine. The decision-making rules are designed to trigger alerts when specific labels, such as “no_helmet” or “without_helmet,” are detected. In the absence of these labels, even valid detections (e.g., “person”) do not contribute to PPE violation detection. This highlights the dependency of the system’s performance on the availability of correctly trained and labeled models.

Furthermore, environmental conditions such as lighting, camera positioning, and object visibility also influence detection performance. In low-light or occluded environments, the confidence scores of detections may fall below the defined threshold, resulting in filtered or discarded outputs.

To ensure consistent testing during development, simulated sensor data was used alongside the vision module. While this approach allows for controlled evaluation of the fusion logic, it may not fully represent real-world variability, particularly in the absence of a fully trained PPE detection model.

# CHAPTER 5: Results and Discussion

Interpretation of Results

The hybrid approach consistently improves operational reliability by corroborating visual detections with sensor telemetry. Fusion reduces false positives from transient sensor spikes and mitigates vision failures due to occlusion or poor lighting. Edge optimization techniques enable near-real-time inference while keeping power and thermal footprints manageable. [6][11][18]

Tradeoffs and Design Choices

Key tradeoffs include accuracy versus latency and privacy versus observability. Larger models yield higher accuracy but increase inference time; quantization and pruning reduce latency at some cost to accuracy. On-device processing preserves privacy, but limits centralized analytics; selective upload of anonymized event summaries balances these concerns.

Deployment Challenges and Mitigations

Practical deployment challenges include sensor calibration drift, network outages, and worker acceptance. Mitigations include scheduled calibration routines, local buffering with eventual sync, transparent communication and opt-in policies for monitored personnel, and configurable alert thresholds to reduce nuisance alarms.

Ethical and Regulatory Considerations

Ethical deployment requires clear policies on data retention, access control, and non-punitive use of monitoring data. Anonymization, face blurring, and strict role-based access to footage are recommended. Compliance with workplace safety regulations and data protection laws must be verified for each deployment jurisdiction.

CHAPTER 6: Conclusions

Summary of Contributions

This dissertation will present a comprehensive proposal for a hybrid Internet of Things (IoT) and Computer Vision framework designed to enhance real-time industrial hazard detection and worker safety compliance. The anticipated contributions of this research will include: [11][17][20]

- The design of a modular system architecture that integrates environmental sensing, visual analytics, edge computing, and real-time alerting.

- The development of an edge-optimized computer vision pipeline capable of detecting personal protective equipment (PPE) violations and unsafe worker behaviors with high accuracy and low latency.

- The implementation of a sensor–vision fusion mechanism that combines environmental telemetry with visual evidence to improve decision confidence and reduce false positives.

- A structured evaluation plan that will assess system performance under controlled and semi-operational conditions, focusing on detection accuracy, responsiveness, and operational reliability.

Through these contributions, the research will aim to demonstrate the feasibility and effectiveness of intelligent safety monitoring systems aligned with Industry 4.0 principles.

Fig. AI Applications for the Construction Industry

(Src : https://sharpml.com/ai-for-construction-061720/)

Practical Implications

The proposed system is expected to offer significant practical benefits for industrial stakeholders. By automating the detection of safety violations and environmental hazards, the framework will provide a proactive mechanism for reducing workplace accidents and ensuring regulatory compliance. The integration of sensor–vision fusion will enhance the reliability of alerts, minimize false alarms and enable prioritized responses to critical events. Real-time monitoring and edge-based processing will support rapid intervention, while the dashboard interface will facilitate actionable insights for safety officers and plant managers. Overall, the system will serve as a scalable and adaptable solution for modern industrial environments seeking to improve occupational safety through intelligent automation.

CHAPTER 7: Limitations

While the proposed framework is designed to be robust and scalable, several limitations are anticipated:

- Dataset Dependency: The accuracy of computer vision models will depend heavily on the quality and diversity of the training dataset. Limited representation of real-world scenarios may affect generalization.

- Environmental Sensitivity: Performance may degrade under extreme conditions such as poor lighting, heavy occlusion, or sensor interference, which are common in industrial settings.

- Sensor Calibration and Maintenance: The reliability of sensor data will require regular calibration and maintenance. Drift or hardware failure could compromise detection accuracy and system responsiveness.

- Edge Resource Constraints: Despite optimization, edge devices may face computational or thermal limitations that restrict model complexity or continuous operation.

These limitations will be documented and addressed through mitigation strategies during the implementation and evaluation phases.

CHAPTER 8: Suggestions and Future Scope

Building upon the proposed framework, several avenues for future research and development will be explored:

- Multi-Camera Fusion: To address occlusion and improve spatial coverage, future iterations of the system may incorporate multi-camera setups with synchronized inference and cross-view tracking.

- Federated Learning: To enhance model performance across diverse industrial sites while preserving data privacy, federated learning techniques will be investigated. This will allow decentralized training using local data without sharing raw inputs.

- Adaptive Thresholding: The fusion engine may be extended with online learning capabilities to dynamically adjust detection thresholds based on historical patterns and feedback, thereby reducing false alarms over time.

- Enterprise Integration: The system will be designed for compatibility with enterprise safety management platforms, enabling automated incident logging, compliance reporting, and integration with broader industrial control systems.

- Explainable AI (XAI): Future work may also incorporate interpretability mechanisms such as visual heatmaps or decision rationales to improve user trust and facilitate human-in-the-loop validation.

These directions will support the continued evolution of intelligent safety systems and contribute to the broader adoption of AI-driven solutions in industrial risk management.

CHAPTER 9: References / Bibliography

Journal / Conference Papers:

- Awalgaonkar, N.M., Zheng, H., Gurciullo, C.S., “DEEVA: A Deep Learning and IoT Based Computer Vision System to Address Safety in Industrial Sites,” arXiv, 2020.

- Wu, J., Wu, S., Ma, Y., “MonitorVLM: A Vision-Language Framework for Safety Violation Detection in Mining Operations,” arXiv, 2025.

- Khandelwal, P., Khandelwal, A., Agarwal, S., “Using Computer Vision to Enhance Safety of Workforce in Manufacturing Industries,” arXiv, 2020.

- Eiffert, S., Wendel, A., Colborne-Veel, P., “Toolbox Spotter: A Computer Vision System for Real-World Situational Awareness in Heavy Industries,” arXiv, 2021.

- Uppaluri, V.R., “Real-Time AI-Driven Hazard Detection: Integrating Computer Vision and Sensor Networks for Enhanced Mining Safety,” International Journal of Scientific Research in Computer Science, Engineering, and Information Technology, 2025.

- Lan, R., et al., “Computer Vision for Safety Management in the Steel Industry,” MDPI, 2024.

- “An Evaluation of the Technologies Used for the Real-Time Monitoring of the Risk of Falling from Height in Construction — Systematic Review,” MDPI, 2024.

- Yousif, I., et al., “Safety 4.0: Harnessing Computer Vision for Advanced Manufacturing Safety,” ScienceDirect, 2024.

- “Artificial Intelligence in Manufacturing Industry Worker Safety: A New Paradigm for Hazard Prevention and Mitigation,” MDPI, 2023.

- Park, J., et al., “Artificial Intelligence and Smart Technologies in Safety Management: Real-Time Surveillance Using IoT Sensors and Computer Vision,” MDPI, 2024.

- Kalekar, A., Kadam, R., Tamhane, S., “Hybrid AI-IoT System for Real-Time Worker Safety and Hazard Detection,” International Journal of Scientific and Advanced Technology, 2025.

- Ejaz, U., Ramon, W., Blessing, M., “IoT for Hazard Detection and Worker Safety Monitoring,” ResearchGate, 2025.

- Ozobu, C.O., Adikwu, F.E., et al., “Advancing Occupational Safety with AI-Powered Monitoring Systems: A Conceptual Framework for Hazard Detection and Exposure Control,” IIARD Journals, 2025.

- “Real-Time Worker’s Safety Monitoring on Scaffolding: Integrating IoT with Computer Vision,” International Research Journal of Advanced Engineering and Technology, 2025.

- “Fall Prevention from Scaffolding Using Computer Vision and Smart Safety Hooks,” ASCE Library, 2025.

- Konda, R., “Enhancing Workplace Safety with AI-Powered Video Surveillance and Real-Time Hazard Detection,” Quest Journals Journal of Scientific and Engineering Research, 2020.

- “The Improvement of Industrial Safety Measures Using AI and IoT,” Journal of Advanced Science and Technology, 2025.

- “Enhancing Industrial Safety and Risk Assessment Through AI-Powered Monitoring,” ESR Groups Journal, 2024.

- Asif, M.J., Saqib, S., Ahmad, R.F., et al., “Leveraging Wireless Sensor Networks for Real-Time Monitoring and Control of Industrial Environments,” arXiv, 2025.

- Huber, J., et al., “Evaluating User Safety Aspects of AI-Based Systems in Occupational Environments,” PMC, NCBI, 2025.